# Connect SatLoc Bantam to Reabe Smart Gate.

Rev A

Gate Software A0  08 or Higher

Satloc Software V 5.5.31.5 or Higher

GPS Paint wires can be connected to Pins 5 and 6 of Open Close Switch.

Wire GPS to Controller:

| Concetion | Wire Reabe 232 GPS |  | Conection | Wire Satloc | Plug Pin |
| --- | --- | --- | --- | --- | --- |
| Gnd Reabe | Shield | >-> | GND | GRN or SH | 5 |
| 232 In | Wht/Blue | >-> | 232 TX | RED | 3 |
| 232 Out | White | >-> | 232 RX | WHT | 2 |

Cable End if Desired.

What lines will he need to tie into to get the Speed data out.

You have a couple choices here.

- You can purchase one of our cables, but you’ll likely want to cut off the other end and replace it with a connector of your choosing.  Such a cable can ship today.

- You can purchase the CONXALL 3282-8PG-321 and wire it up as below.  The RTS/CTS are not needed.

Here are some links to that connector:

https://www.eciaauthorized.com/en/part/conxall/3282-8PG-321

https://www.sager.com/3282-8pg-321-1107420.html

https://www.digikey.com/product-detail/en/conxall-switchcraft/3282-8PG-321/SC3158-ND/3514169

Hello David,

I am sending you the GPS connection Data so you have it all in one place.

The software is at:

https://www.reabeair.com/wp-content/uploads/2021/11/AirBantam_5.5.31.5.zip

A hook up guide is at:

https://www.reabeair.com/wp-content/uploads/2021/11/Connect-SatLoc-Bantam-to-Reabe-Smart-Gate-REV-1.docx

I did send the wire info.

Satloc is checking in on the TX and RX lines.

Because a serial Connection goes as shown:

GPS----Gate

TX--->---RX

RX---<---TX

They did not know if the TX Label is for the GPS or the Gate.

On the G4,

The White line is there RX line that goes to my White TX line

The Red line is there TX line that goes to my Wht/Blue RX line

I would guess it to be the same for the Bantam.